- B.Eng.Composite Materials Engineering

- Health and Safety

- Control of Substances Hazardous to Health (COSHH)

- Can you substitute a safer material

Can you use an engineering solution

Are you using appropriate personal protection

- Environmental Protection Act

- You must dispose of materials in the correct manner

- A clean and tidy workplace is a safer workplace

- If you are not using it put it away

- If you must leave anything out, make sure it is clearly labelled

- Health and Safety: Unsaturated PolyEster resin (UPE)

- Base resin:  polymer chains with reactive sites (50%)

- Styrene:  reactive diluent (50%)

volatile

flammable:

flash point 31°C

explosive limits 1.1-8.0%

NO SMOKING, no naked flames

toxic:

(inhalation)	odour threshold is 25 ppm

(the Scandinavian Occupational Exposure Limit)

little inhalation irritation below 210 ppm

(UK OEL is 100 ppm 8h TWA, 250 ppm 10 min TWA)

(UK voluntary code is 50 ppm 8h TWA (time weighted average))

IRRITATING to nose and throat above 500 ppm

(ingestion)	IRRITATING to mouth, throat and stomach

may lead to vomiting and dizziness

harmful effects at >4 g/kg of body weight

(skin contact)	IRRITATING: evident as itching and redness

frequent or prolonged contact leads to dermatitis

degreases skin

(eye contact)	irritation may last several hours

COSHH substitution:	resin without styrene (eg epoxy) or low styrene emission resin

engg controls	general ventilation to reduce background level

local exhaust ventilation with optimised flow away from operator

use shielded rollers to reduce droplet cloud

protection	lab coat, barrier cream, gloves, goggles

Disposal	to waste solvent bottle if resin uncured

- Health and Safety: Unsaturated PolyEster resin (UPE)

Peroxide Initiator (usually added as 1-2%)	AAP:	Acetyl acetone peroxide

BzO2	Benzoyl peroxide

H2O2	Hydrogen peroxide

MEKP	Methyl ethyl ketone peroxide

SEVERE IRRITANTS to skin

CAUSE BURNS

wash immediately for 15 minutes with water,  obtain doctors attention

CORROSIVE to moist tissue (eyes, nose, throat, airways to lungs)

avoid contact between MEKP and rust

Irreversible damage may be caused to eyes by prolonged contact

BzO2: EXTREME RISK OF EXPLOSION by shock, friction, fire or ignition

COSHH substitution: different resin system?

engg controls:	use as a dilute solution if practicable

protection:	protective clothing, face/eye protection

handle and open container with care

Ignition		contact with combustible material may cause fire

do not mop up with eg paper cloths

Disposal		do not empty into drains

- Accelerators (1-2%)

- Cobalt based (Co-naphthenate, Co-octoate) usually as solutions in styrene

- Amine based accelerators ... up to 25% solution in styrene

TOXIC if swallowed, inhaled or allowed to remain in contact with the skin

FUMES from burning accelerators contain toxic materials

- NEVER

mix initiator directly with accelerator

they will form an EXPLOSIVE mixture

- Health & Safety: Epoxy resin (Ep)

Base resin		Mild to moderate primary skin irritants

Irritation potential increased by prolonged skin contact

EPOXY SENSITIZATION

reddening of skin even in close proximity to uncured resin

Curing agents		For all classes of curing agent:

protective clothing should be worn

goggles should be worn

ventilation is essential

- Aliphatic amines

alkaline caustic materials

cause burns

severe tissue damage to skin, mucous membranes and eyes

ALL contact should be avoided

- Solid aromatic amines

less caustic, less irritating and less sensitising than aliphatic amines

Diaminodiphenylmethane (DAPM) is a toxic chemical

known to cause liver damage in humans

can be absorbed through the skin: all contact should be avoided

- Cycloaliphatic amines

variable in their irritation and sensitising effect

extremely irritating to the eyes

- Polyamides

skin irritants of varying sensitivity,

but generally non-sensitising to the skin

extremely irritating to the eyes

- Health & Safety: reinforcement fibres

- All reinforcements should be regarded as a nuisance dust when cutting/machining

Control limits are	10 mg/m3	total dust

5 mg/m3	man-made mineral fibre

- Respirable dust is believed to have a particle size of <3 μm

and for safety a value of 5 μm is assumed

- The major reinforcement fibres have diameters in the range 6-15 μm

carbon and glass fibres are not expected to break into lengths shorter than the diameter

aramid (and probably polyethylene) fibres have a complex microstructure

and may fibrillate into particles of <3 μm

and may therefore be respirable!

- Disposal

All man-made fibres are a notifiable waste and must be disposed of into the bagged bins provided

- Health & safety: summary

Remember to:

- Read and follow the supplier's safety sheet and instructions for use

- Store and handle materials in the appropriate safe way

- Consider alternative materials and engineering controls

- Use ventilation and fume/dust control equipment properly

- Wear appropriate personal protection

- Good housekeeping is essential for your safety and that of your colleagues

And do not:

- Smoke, eat or drink in the laboratory or storage areas

- Mix peroxide initiator with accelerators

- Allow waste to accumulate

- Use solvents for cleaning skin

Use combustible materials to soak up spillage